| - ERTMS/ATO |  |
| --- | --- |
| ATO-OB / ATO-TS FFFIS Application Layer |  |
| REF : | SUBSET-126 version updated for including GoA3-4 operations |
| ISSUE : | N.A. |
| DATE : | N.A. |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| 0.0.1 07-01-2014 | All | First issue of the document | UNISIG ATO WP |
| 0.0.2 08-04-2014 | All | Document updated after UNISIG ATO WG comments and review meetings (11-12/02/2014 and 11-12/03/2014). | UNISIG ATO WP |
| 0.0.3 30-04-2014 | All | Document updated after UNISIG ATO WG comments. | UNISIG ATO WP |
| 0.0.4 06-05-2014 | All | Document updated after UNISIG ATO WG comments to version 0.0.3 and call conference of 05/05/2014. | UNISIG ATO WP |
| 0.0.5 05-09-2014 | All | Document updated after EUG review and SUBSET-125-006 and 007 discussions and update. | UNISIG ATO WP |
| 0.0.6 29-09-2014 | All | Document updated after UNISIG ATO WG review. All the requirements and explanation have been deleted from the document. | UNISIG ATO WP |
| 0.0.7 13-05-2015 | All | The document content has been updated according to the EUG and ERA comments to version 0.0.5 and the SUBSET-125 update to version 0.0.10. | UNISIG ATO WP |
| 0.0.8 07-01-2016 | Table 7, Table 8, Table 9 | Minor changes according to UNISIG ATO WG Workshops of May, June and July 2015. | UNISIG ATO WP |
| 0.0.9 11-04-2016 | All | Document updated after UNISIG ATO WG review. | UNISIG ATO WP |
| 0.0.10 19-07-2016 | 5.1, 6.3.3, 6.3.4, 6.3.5 | Document updated after UNISIG ATO WG review. | UNISIG ATO WP |
| 0.0.11 25-08-2016 | 5.1, 5.2, 6.3.3.2, 6.3.4.2, 6.3.5.2 | Document updated after UNISIG ATO WG review. | UNISIG ATO WP |
| 0.0.12 27-04-2017 | All | Modification according to UNISIG SG, EUG and ATO WG review. | UNISIG ATO WP |
| 0.0.13 19-05-2017 | All | Modification according to ATO WG review. | UNISIG ATO WP |
| 0.0.14 draft A 15-12-2017 | All | Modification to align with SUBSET-125-0017 draftC. | UNISIG ATO WP |
| 0.0.14 22-12-2017 | 7.2.1.2, 7.3.6 | Modification to align with SUBSET-125-0017. | UNISIG ATO WP |
| 0.0.15 16-03-2018 | 7.2.1.2, 7.3.8.2 | Modification following ATO-WP review on version 0.0.14 and alignment with SUBSET-125-0018. Change of N_ITER Description and Resolution / Formula. Minor spelling corrections. | UNISIG ATO WP |
| 0.0.16 07-05-2018 | All | Modification to align with SUBSET-125-010 and SUBSET-130-010. Offsets replaced by Item Number. | UNISIG ATO WP |
| 0.0.17 05.08.2020 | All | Review comments and following S2R comments against Subset-126 solved: 1, 2, 6, 7, 8, 9, 10, 12, 13, 14, 16, 19, 22, 23, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 45, 50, 54, 55, 57, 58, 59, 64, 66, 67, 68, 71, 72, 74, 76, 77, 79, 80, 82, 83, 84, 85, 86, 87, 88, 89, 91, 92, 93, 94, 98, 100, 101, 103, 104, 105, 106, 107, 108, 109, 110, 111 | UNISIG ATO WP |
| 0.0.18 22.10.2020 | All | Following S2R comments solved: 114, 115, 137 Following EECT comments solved: 1, 2, 3, 4, 9, 12, 13, 14, 15, 16, 20, 23, 24, 25, 26, 29, 32, 36, 37 | UNISIG ATO WP |
| 0.0.19 02.11.2020 | All | Following EECT comments solved: 5, 6, 7 | UNISIG ATO WP |
| 0.0.20 17.01.2021 | All | Following EECT comments solved: 10 | UNISIG ATO WP |
| 0.0.21 25.06.2021 | 6.1.1.1. 6.1.1.2. 6.2.1.8. 6.2.1.9. 5.2. 7.3.10.2. | Implementing Consolidation phase comments #1 to #4 | UNISIG ATO WP |
| 0.0.22 16.09.2021 | 6.2.1.10 6.3.1.5 7.3.6.2 item 31 7.3.9.2 item 13 | Implementing comments from SUBSET-126-0.0.26.docx Date: 18/08/21 | UNISIG ATO WP |
| 0.0.23 08.11.2021 | Table 3, item 2, 3 Table 7. item 63 Table 8, item 11, 27, 28 Table 11, items 8, 51, 56 Table 13, item 18 | Implementing comments from SUBSET-126-0.0.26.docx Date: 21/09/21 (EECT051021) | UNISIG ATO WP |
| 0.0.24 22.11.2021 | Tables 8, 11, 12 | Implementing agreed comments in document “ATO Document consolidated review sheet – Batch 6 -21 11 15” | UNISIG ATO WP |
| 0.0.25 26.01.2022 | several | ATO over ETCS modified to ERTMS/ATO N_ITER_ATO changed to N_ITER, N_ITER for Balise Groups modified to N_ITER_BG (3bit), explanation added for ETRS89 | UNISIG ATO WP |
| 0.0.26 07.02.2022 | several | Modifications to N_ITER_ATO aligned to agreed comment #5 from “ATO Document consolidated review sheet - Batch 6_EECT150222” Description of N_ITER_BG for balises aligned to comment #2 of document “ATO Document consolidated review sheet - Batch 5_EECT071221” Description of M_Dynamic_Brake_Force_Limit according to comment #2 of document “ATO Document consolidated review sheet - Batch 5_EECT071221” | UNISIG ATO WP |
| 0.1.0 24.03.2022 | several | Editorial corrections | UNISIG ATO WP |
| 0.1.1 07.02.2023 | 7.3.13.2 (SESSTerm packet, item 2) | “CAB closed” is replaced by “CAB inactive” according the comment #1 related to the SUBSET-126 from EECT#91 | UNISIG ATO WP |
| 0.1.2 07.04.2023 | 7.3.2.2 7.3.9.2, line #56 7.3.3.2. Line #5 Footer | Adding the following attributes to a handshake request: Q_ATO_Handing over N_ITER_BG changed from 3 bits to 4 bits. The definition has been updated according to the comment #5 related to the SUBSET-125 in the EECT 3rd consolidation review sheet. Version number updated | UNISIG ATO WP |
| 1.0.0 05.07.2023 | None | Baseline 1 1st release | UNISIG ATO WP |
| Not applicable D6.5 Annex3 | 7.3.2, 7.3.3, 7.3.6, 7.3.9 | Update of values of variable M_ATO_VERSION Updates for doors operation in GoA3-4 Update for GoA3/4 constraints | R2DATO task 6.5 |

# Table of Contents

1.	Modification History	2

2.	Table of Contents	6

3.	Table of Figures	8

4.	List of Tables	9

5.	Introduction	10

5.1.	Scope and purpose of the document	10

5.2.	Reference and applicable documents	10

5.3.	Abbreviations	11

5.4.	Definitions	11

6.	Principles	12

6.1.	Introduction	12

6.2.	Definition of the Variables	12

6.3.	Definition of the Packets	13

7.	Detailed Description of the Application Level	14

7.1.	List of Packets	14

7.2.	ATO Header	14

7.3.	ATO Packets Applicable Content	15

7.3.1.	Introduction	15

7.3.2.	Handshake Request Packet (HSReq)	15

7.3.3.	Handshake Acknowledgement Packet (HSAck)	16

7.3.4.	Handshake Reject Packet (HSRej)	17

7.3.5.	Journey Profile Request Packet (JPReq)	17

7.3.6.	Journey Profile Packet (JP)	18

7.3.7.	Journey Profile Acknowledgement Packet (JPAck)	23

7.3.8.	Segment Profile Request Packet (SPReq)	23

7.3.9.	Segment Profile Packet (SP)	24

7.3.10.	Status Report Packet (STR)	40

7.3.11.	Status Report Acknowledgement Packet (STRAck)	42

7.3.12.	Session Termination Request Packet (SESSTermReq)	43

7.3.13.	Session Termination Packet (SESSTerm)	43

Appendix	45

A.1. 	Example Encoded Handshake Request Packet	45

A.1.1. 	The following example shows the encoding of a Handshake Request Packet.	45

# Table of Figures

Figure 1 Encoded Handshake Request Packet	45

# List of Tables

Table 1 Reference and applicable documents	10

Table 2 Packet summary	14

Table 3 ATO Header structure	15

Table 4 Handshake Request Packet structure	16

Table 5 Handshake Acknowledgement Packet structure	16

Table 6 Handshake Reject Packet structure	17

Table 7 Journey Profile Request Packet structure	17

Table 8 Journey Profile Packet structure	22

Table 9 Journey Profile Acknowledgement Packet structure	23

Table 10 Segment Profile Request Packet structure	24

Table 11 Segment Profile Packet structure	37

Table 12: Type Classification for Segment Profile Data	39

Table 13 Status Report Packet structure	42

Table 14 Status Report Acknowledgement Packet structure	43

Table 15 Session Termination Request Packet structure	43

Table 16 Session Termination Packet structure	44

# Introduction

## Scope and purpose of the document

#### The purpose of this System Interface Description document is to present the interoperable interface between the two subsystems of the Automatic Train Operation (ATO) system, namely the ATO trackside (ATO-TS) and the ATO on-board (ATO-OB). The scope of the document is to define the interoperable packets exchanged between the on-board subsystem (ATO-OB) and the trackside subsystem (ATO-TS) according to the ERTMS/ATO System Requirements Specification [Ref 1].

#### This document does not contain the communication requirements associated to the ATO-OB / ATO-TS Interface. These requirements are included in [Ref 1]§10.1.

#### Specific ATO Packets are defined in this document with a unique packet identification number.

## Reference and applicable documents

#### This section presents the reference and applicable documents.

| Ref. N° | Title | Reference |
| --- | --- | --- |
|  | ERTMS/ATO System Requirements Specification | SUBSET-125 |
|  | ERTMS/ETCS System Requirements Specification | SUBSET-026 |
|  | ERTMS/ATO Glossary | 13E154 |
|  | FFFIS STM Application Layer | SUBSET-058 |
|  | Glossary of Terms and Abbreviations | SUBSET-023 |
|  | FIS Juridical Recording | SUBSET-027 |
|  | Responsibilities and Rules for the Assignment of Values to ETCS Variables | SUBSET-054 |

Table 1 Reference and applicable documents

## Abbreviations

#### For ATO related abbreviations see ERTMS/ATO Glossary [Ref 3].

#### For ETCS related abbreviations see SUBSET-023 [Ref 5].

## Definitions

#### For ATO related definitions see ERTMS/ATO Glossary [Ref 3].

#### For ETCS related definitions see SUBSET-023 [Ref 5].

# Principles

## Introduction

#### The composition of each packet element is presented using a table detailing each variable in order.

## Definition of the Variables

#### Variables are used to encode single data values. Variables cannot be split in minor units.

#### Variables may have special values which are related to the basic meaning of the variable.

#### Special values which are not spare, have always the highest values in a variable (e.g. 32767 = “unknown”).

#### Spare values are located between the normal and special values in the variable range except where justified.

#### Names of variables are unique with respect to their meaning. A variable is used in context with the meaning as described in the variable definition. Variables with different meanings have different names.

#### One bit variables (Boolean) always use 0 for false and 1 for true.

#### Offsets for numerical values are avoided (0 is used for 0, 1 for 1, etc.) except where justified.

#### The encoding order shall respect the order of variables listed in the packet format (from top to bottom).

#### Encoding of variables shall start with the most significant bit. See example in Appendix A.1.

#### Note: A number of variables contain values which have to be assigned. Some of these values have to be unique to ensure that the system functions properly. A centralised handling of this assignment is therefore required (nationally or internationally, depending on the variable). The variables concerned have been marked. The values included in this document for these variables are therefore not to be used without prior verification of their validity. See SUBSET-054 [Ref 7] for further details.

## Definition of the Packets

#### Packets are multiple variables grouped into a single unit, with a defined internal structure.

#### Optional variables inside a packet are marked with constraints. Only if the constraint is evaluated to true the variable and possible nested variables will follow in the packet. Otherwise the packet will continue with the next variable without constraint or with a constraint evaluated to true.

#### An ATO Packet consists of:

#### A specific ATO Header. This header is generated by the subsystem (ATO-OB or ATO-TS) that sends the packet;

#### The Applicable Content itself.

#### Any packet exchanged between ATO-OB and ATO-TS shall add the needed quantity of bits in order to be byte aligned.

##### Note: Thus, the maximum number of potentially added bits is 7 bits.

#### The structure of the HSReq, HSAck and HSRej (packets NID_PACKET_ATO 0, 1 and 2) shall be version independent as they are used for the ATO system version check.

# Detailed Description of the Application Level

## List of Packets

#### This section details the application level of the interface.

| #### Packet Number #### (NID_PACKET_ATO) | #### Packet Name | #### Source | #### Sink | #### Page N° |
| --- | --- | --- | --- | --- |
| #### 0 | #### Handshake Request | #### ATO-OB | #### ATO-TS | #### 15 |
| #### 1 | #### Handshake Acknowledgement | #### ATO-TS | #### ATO-OB | #### 16 |
| #### 2 | #### Handshake Reject | #### ATO-TS | #### ATO-OB | #### 16 |
| #### 3 | #### Journey Profile Request | #### ATO-OB | #### ATO-TS | #### 17 |
| #### 4 | #### Journey Profile | #### ATO-TS | #### ATO-OB | #### 17 |
| #### 5 | #### Journey Profile Acknowledgement | #### ATO-OB | #### ATO-TS | #### 22 |
| #### 6 | #### Segment Profile Request | #### ATO-OB | #### ATO-TS | #### 23 |
| #### 7 | #### Segment Profile | #### ATO-TS | #### ATO-OB | #### 23 |
| #### 8 | #### Status Report | #### ATO-OB | #### ATO-TS | #### 39 |
| #### 9 | #### Status Report Acknowledgement | #### ATO-TS | #### ATO-OB | #### 41 |
| #### 10 | #### Session Termination Request | #### ATO-TS | #### ATO-OB | #### 42 |
| #### 11 | #### Session Termination | #### ATO-OB | #### ATO-TS | #### 42 |

Table 2 Packet summary

## ATO Header

#### This section details the ATO specific header of a packet exchanged between the ATO-OB and the ATO-TS.

#### The information in the header allows each packet to be uniquely identified.

#### ATO Header composition:

| ATO Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Variable Name | Description | Length | Resolution/Formula |
|  | NID_PACKET_ATO | Packet number, used in the header for each packet, allowing the receiving equipment to identify the data which follows. | 8 bits | Numbers Special Values: 12 - 255 = spare |
|  | NID_OPERATIONAL | Train Running Number, see [Ref 2], Section 7.5.1.92. | 32 bits | See [Ref 2], Section 7.5.1.92. |
|  | NID_ENGINE | ETCS Identity, see [Ref 2], Section 7.5.1.88. | 24 bits | See [Ref 2], Section 7.5.1.88. |
|  | T_Timestamp_Date | Date of timestamp of the packet. | 15 bits | Date of a timestamp in UTC. The variable represents the number of days from the 1st January 2010. Values from 0 (01/01/2010) to 32767 (18/09/2099) |
|  | T_Timestamp_Seconds | Seconds of timestamp of the packet in UTC. | 17 bits | Seconds of a timestamp in UTC. The variable represents the number of seconds from the beginning of the day. Values from 0 (00:00:00) to 86399 (23:59:59) |
|  | N_Packet_Counter | Packet counter sent from the sending subsystem. This value is a circulating counter. In case of an overflow the value might jump from the maximum value to the minimum value. The value is maintained by the sender for each NID_PACKET_ATO separately during the entire session. | 8 bits | Binary to numeric |

Table 3 ATO Header structure

## ATO Packets Applicable Content

### Introduction

#### This section defines the actual ATO information contained in the packet.

### Handshake Request Packet (HSReq)

#### This section details the structure of a HSReq Packet sent by ATO-OB to ATO-TS.

#### Handshake Request composition:

| Packet Number |  |  | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  |  | Description | Length | Resolution/Formula |
|  | ATO Header |  |  | see chapter 7.2 |  | - |
| Handshake Details |  |  |  |  |  |  |
|  | N_ITER |  |  | Number of iterations of ATO-OB supported versions. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  | M_ATO_Version (k) |  | ATO system version supported. | 16 bits | 1st byte for the major Version 2nd byte for the minor Version Applicable values: 0x0100 = ATO version 1.0 0x0101 = ATO version 1.1 0x0200 = ATO version 2.0 |
| 004 | Q_ATO_Handing_Over |  |  | This attribute allows the ATO-TS to know if the handshake request is due to a handing over ATO-TS. | 1 bit | 0 = No handing over 1 = Handing over |

Table 4 Handshake Request Packet structure

### Handshake Acknowledgement Packet (HSAck)

#### This section details the structure of the HSAck Packet sent by ATO-TS to ATO-OB.

#### Handshake Acknowledgement composition:

| Packet Number |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |
| Handshake Acknowledgement Details |  |  |  |  |  |
|  | NID_C |  | Identity of the ATO-TS’s country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  | NID_ATOTS |  | Identifier of the ATO-TS. | 14 bits | Binary to numeric |
|  | M_ATO_Version |  | ATO system version to be used. | 16 bits | 1st byte for the major Version 2nd byte for the minor Version Applicable values: 0x0100 = ATO version 1.0 0x0101 = ATO version 1.1 0x0200 = ATO version 2.0 |
|  | T_Timeout_ATOTS_Response |  | Maximum time after which the ATO-OB must consider that a request has not been answered. | 8 bits | Binary to numeric (in seconds) |
|  | T_Reporting_Time |  | Reporting time cycle for triggering a Status Report for data reporting purposes. | 8 bits | Binary to numeric (in seconds) |

Table 5 Handshake Acknowledgement Packet structure

### Handshake Reject Packet (HSRej)

#### This section details the structure of the HSRej Packet sent by ATO-TS to ATO-OB.

#### Handshake Reject composition:

| Packet Number |  | 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |
| Handshake Reject Details |  |  |  |  |  |
|  | Q_Reject_Reason |  | Qualifier indicating the reason for the rejection. | 2 bits | 0 = ATO system version incompatible 1 = Another ATO-TS in charge 2 = ATO-TS in charge unknown 3 = Spare |
| ATO-TS Contact Information - If [ Q_Reject_Reason == Another ATO-TS in charge ] |  |  |  |  |  |
|  | NID_C |  | If [ Q_Reject_Reason == Another ATO-TS in charge ] Identity of the ATO-TS’s country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  | NID_ATOTS |  | If [ Q_Reject_Reason == Another ATO-TS in charge ] Identifier of the ATO-TS. | 14 bits | Binary to numeric |

Table 6 Handshake Reject Packet structure

### Journey Profile Request Packet (JPReq)

#### This section details the structure of a JPReq Packet sent by the ATO-OB to the ATO-TS.

#### Journey Profile Request Packet composition:

| Packet Number |  | 3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |
| Journey Profile Request Details |  |  |  |  |  |
|  | NID_C |  | Identity of the reference SP’s country or region. Not relevant if NID_SP is undefined. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  | NID_SP |  | Identity of the Segment Profile from which a Journey Profile is requested. | 32 bits | Binary to numeric 4294967295 for undefined value. |

Table 7 Journey Profile Request Packet structure

### Journey Profile Packet (JP)

#### The section details the structure of a JP Packet sent by the ATO-TS to the ATO-OB.

#### Journey Profile Packet composition:

| Packet Number |  |  |  | 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  |  |  | Description | Length | Resolution/Formula |
|  | ATO Header |  |  |  | see chapter 7.2 |  | - |
| Journey Profile Details |  |  |  |  |  |  |  |
|  | Q_JP_Status |  |  |  | Status of the Journey Profile: 'Valid': JP containing the data requested. 'Unavailable': JP specifies that the requested part of the Journey Profile is currently not available yet (but all the previously sent JPs are still applicable). 'Invalid': JP specifies that the SP identifier asserted in the JPReq does not belong to the preceding JP already sent to the ATO-OB. 'Update': JP specifies that the Journey Profile has been updated by the TMS within the current visibility of the ATO-OB. 'Overwrite': JP specifies that the previously sent JPs shall be completely overwritten by this data. | 3 bits | 0 = Invalid 1 = Valid 2 = Unavailable 3 = Update 4 = Overwrite [5-7] = Spare |
| Segment Profile Reference - If [ Q_JP_Status <> Invalid and Q_JP_Status <> Unavailable ] |  |  |  |  |  |  |  |
|  | N_ITER_SP |  |  |  | If [ Q_JP_Status <> Invalid and Q_JP_Status <> Unavailable ] Number of iterations of SPs. If N_ITER_SP is 0 then no data set is following. | 8 bits | Binary to numeric |
|  |  | NID_C (k) |  |  | Identity of the SP’s country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  | NID_SP (k) |  |  | SP identity. | 32 bits | Binary to numeric |
|  |  | M_SP_Version (k) |  |  | Identifier of the segment profile version. See section Segment Profile Packet (SP). | 16 bits | 1 byte for the major version and 1 byte for the minor one. |
|  |  | Q_SP_DIR (k) |  |  | Qualifier to indicate the valid travelling direction of the SP. | 1 bits | 0 = Reverse 1 = Nominal |
| Timing Point Constraints |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  | Number of iterations of TPs information. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | NID_TP (k,l) |  | TP identity. The NID_TP is unique within the NID_C of the Segment Profile. | 32 bits | Binary to numeric. |
|  |  |  | T_Latest_Arrival_Date (k,l) |  | Date of the requested arrival time at the TP. | 15 bits | Date of a timestamp in UTC. The variable represents the number of days from the 1st January 2010. Values from 0 (01/01/2010) to 32767 (18/09/2099) |
|  |  |  | T_Latest_Arrival_Seconds (k,l) |  | Seconds of the requested arrival time at the TP. | 17 bits | Seconds of a timestamp in UTC. The variable represents the number of seconds from the beginning of the day. Values from 0 (00:00:00) to 86400 (24:00:00) |
|  |  |  | T_Arrival_Window (k,l) |  | Acceptable time allowance to be earlier at the TP. | 10 bits | Binary to numeric (in seconds). The value is 0 for Stopping Points. |
|  |  |  | Q_TP_Alignment (k,l) |  | This qualifier defines if the TP location is applicable from the front, middle or rear of the train. | 2 bits | 0 = Front 1 = Middle 2 = Rear 3 = Spare |
|  |  |  | Q_Stop_Skip_Pass (k,l) |  | Specifies if the Timing Point is a Passing Point, an operational Stopping Point or a skipped Stopping Point. | 2 bits | 0 = Stopping Point 1 = Stopping Point to be skipped (with symbol shown to driver) 2 = Passing Point 3 = Spare |
|  |  |  | Q_TP_Information (k,l) |  | Specifies some information specific for the TP. | 2 bits | 0 = No specific information 1 = End of Journey [2-3] = Spare |
|  |  |  | Q_Day_Light_Saving (k,l) |  | This variable defines if the day light saving hour is applicable to calculate the local time. | 1 bits | 0 = No saving hour 1 = Saving hour |
| Stopping Point Information - If [ Q_Stop_Skip_Pass == Stopping Point ] |  |  |  |  |  |  |  |
|  |  |  | Q_Opening_Door_Side (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point ] Specifies if the ATO-OB has to manage the train doors opening and on which side the passenger exchange doors have to be opened. The side is relative to the direction of the Journey Profile. | 2 bits | 00 = none 01 = right 10 = left 11 = both |
|  |  |  | Q_Centralised_Opening (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point ] This variable defines if the doors are to be opened centralised or by the passengers. | 1 bits | 0 = Opening by passengers 1 = Centralised automatic opening of the relevant doors |
|  |  |  | Q_Relaxed_Coupler (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point ] This variable requests the train to stop with couplers relaxed. | 1 bits | 0 = no request for coupler relaxation 1 = request for coupler relaxation |
| Stopping Point Departure Details - If [ Q_Stop_Skip_Pass == Stopping Point and Q_TP_Information <> End of Journey ] |  |  |  |  |  |  |  |
|  |  |  | Q_Train_Hold (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point and Q_TP_Information <> End of Journey ] The variable defines if the train is requested to be held at the Stopping Point or not. | 1 bits | 0 = Do not hold Train 1 = Hold train |
|  |  |  | T_Departure_Date (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point and Q_TP_Information <> End of Journey and Q_Train_Hold <> Hold train ] Date of the expected departure time from the Stopping Point. | 15 bits | Date of a timestamp in UTC. The variable represents the number of days from the 1st January 2010. Values from 0 (01/01/2010) to 32767 (18/09/2099) |
|  |  |  | T_Departure_Seconds (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point and Q_TP_Information <> End of Journey and Q_Train_Hold <> Hold train ] Seconds of the expected departure time from the Stopping Point. | 17 bits | Seconds of a timestamp in UTC. The variable represents the number of seconds from the beginning of the day. Values from 0 (00:00:00) to 86399 (23:59:59) |
|  |  |  | T_Minimum_Dwell_Time (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point and Q_Train_Hold <> Hold train ] Minimum dwell time at given Stopping Point (in seconds). | 10 bits | Binary to numeric (in seconds) |
|  |  |  | Q_Automatic_Closing (k,l) |  | If [ Q_Stop_Skip_Pass == Stopping Point and Q_Train_Hold <> Hold train ] This variable defines if the ATO-OB has to manage the train doors Closing. | 1 bits | 0 = ATO-OB does not manage train doors closing 1 = ATO-OB manages train doors closing |
| Temporary Constraints |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  | Number of iterations of Temporary Constraints. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_TC_Type (k,l) |  | Type of the temporary constraint. | 3 bits | 0 = ASR 1 = Low Adhesion 2 = ATO Inhibition Zone 3 = DAS Inhibition Zone 4 = Current Limitation 5 = GoA3 Inhibition Zone 6 = GoA4 Inhibition Zone [7] = Spare |
|  |  |  | Q_Range (k,l) |  | Specifies if the temporary constraint starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_TC_Start_Location (k,l) |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Start location of the temporary constraint relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_TC_End_Location (k,l) |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] End location of the temporary constraint relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Additional Speed Restriction - If [ Q_TC_Type == ASR ] |  |  |  |  |  |  |  |
|  |  |  | Q_FRONT (k,l) |  | If [ Q_TC_Type == ASR ] See [Ref 2], Section 7.5.1.109 | 1 bits | See [Ref 2], Section 7.5.1.109 |
|  |  |  | V_Speed_Level (k,l) |  | If [ Q_TC_Type == ASR ] Value of the speed level restriction. The variable used is comparable to V_STATIC, as defined in [Ref 2], Section 7.5.1.171, hence only the first 120 values provided by the 7 bits are used to determine that speed Level. | 7 bits | [0..120] x 5 km/h <=> [0..600] km/h; [121-127] = Spare |
| Low Adhesion - If [ Q_TC_Type == Low Adhesion ] |  |  |  |  |  |  |  |
|  |  |  | Q_Adhesion_Category (k,l) |  | If [ Q_TC_Type == Low Adhesion ] Adhesion category aligned with the definition in EN15595 of the wheel-rail interface for the lower boundaries. Dry Rail: Conditions where 100% of the brake force of the vehicle can be applied with no axle sliding of more than 2% (adhesion level typically above 0.15µ) Medium: Conditions where the wheel/rail adhesion is in the range 0.15 – 0.10 (Damp rails with some contamination) Normal Low: Conditions where the wheel/rail adhesion is in the range 0.10 – 0.08 (Typical autumn mornings due to dew/dampness often combined with light overnight rust) Low Adhesion: Conditions where the wheel/rail adhesion is in the range 0.08 – 0.05 Very Low Adhesion: Conditions where the wheel/rail adhesion is in the range 0.05-0.03 Extremely Low Adhesion: Conditions where the wheel/rail adhesion is below 0.03 | 3 bits | 0 = Dry Rail 1 = Dry Rail (Medium) 2 = Dry Rail (Low) 3 = Low Adhesion 4 = Very Low Adhesion 5 = Extremely Low Adhesion [6-7] = Spare |
| Current Limitation - If [ Q_TC_Type == Current Limitation ] |  |  |  |  |  |  |  |
|  |  |  | M_CURRENT (k,l) |  | If [ Q_TC_Type == Current Limitation ] See [Ref 2], Section 7.5.1.62.1 Note: a powerless section is indicated as a current limit “0” (zero) | 10 bits | See [Ref 2], Section 7.5.1.62.1 |

Table 8 Journey Profile Packet structure

#### If the status of the JP is invalid or unavailable, the Journey Profile Details shall finish after Q_JP_Status (Item 002).

#### Temporary Constraints

##### The location dependent data type of the Temporary Constraints is ATO Area Type. The requirements as given in chapter 7.3.9.7 shall be applied.

##### For each type of constraint, the Temporary Constraints in a Journey Profile shall be listed in the order given by their starting location in the Segment Profile.

### Journey Profile Acknowledgement Packet (JPAck)

#### This section details the structure of a Journey Profile Acknowledgement Packet sent by the ATO-OB to the ATO-TS in response to an updated/overwritten Journey Profile Packet.

#### Journey Profile Acknowledgement Packet composition:

| Packet Number |  | 5 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |
| Journey Profile Acknowledgement Details |  |  |  |  |  |
|  | T_JP_Reference_Timestamp_Date |  | Date of the timestamp of the Journey Profile Packet (see 7.2 ATO Header) for which the acknowledgement is provided. | 15 bits | Date of a timestamp in UTC. The variable represents the number of days from the 1st January 2010. Values from 0 (01/01/2010) to 32767 (18/09/2099) |
|  | T_JP_Reference_Timestamp_Seconds |  | Seconds of the timestamp of the Journey Profile Packet in UTC (see 7.2 ATO Header) for which the acknowledgement is provided. | 17 bits | Seconds of a timestamp in UTC. The variable represents the number of seconds from the beginning of the day. Values from 0 (00:00:00) to 86399 (23:59:59) |
|  | N_JP_Reference_Packet_Counter |  | N_Packet_Counter of the Journey Profile Packet (see 7.2 ATO Header) for which the acknowledgement is provided. | 8 bits | Binary to numeric |

Table 9 Journey Profile Acknowledgement Packet structure

### Segment Profile Request Packet (SPReq)

#### The section details the structure of a SPReq Packet sent by the ATO-OB to the ATO-TS.

#### Segment Profile Request Packet composition:

| Packet Number |  |  | 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  |  | Description | Length | Resolution/Formula |
|  | ATO Header |  |  | see chapter 7.2 |  | - |
| Segment Profile Request Details |  |  |  |  |  |  |
|  | N_ITER |  |  | Number of iterations of SPs requested. The minimum value for the variable in this packet is 1. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  | NID_C (k) |  | Identity of the SP's country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  | NID_SP (k) |  | Identity of the requested Segment Profile. | 32 bits | Binary to numeric |

Table 10 Segment Profile Request Packet structure

### Segment Profile Packet (SP)

#### This section details the structure of a Segment Profile Packet sent by the ATO-TS to the ATO-OB.

#### Segment Profile Packet composition:

| Packet Number |  |  |  |  | 7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  |  |  |  | Description | Length | Resolution/Formula |
|  | ATO Header |  |  |  |  | see chapter 7.2 |  | - |
| Segment Profile Status |  |  |  |  |  |  |  |  |
|  | N_ITER |  |  |  |  | Number of iterations of SPs in the packet. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  | NID_C (k) |  |  |  | Identity of the SP's country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  | NID_SP (k) |  |  |  | SP identity. | 32 bits | Binary to numeric |
|  |  | Q_SP_Status (k) |  |  |  | Status of the Segment Profile: "Valid": SP requested. "Invalid": SP not found in ATO-TS database. | 1 bits | 0 = Invalid 1 = Valid |
| Segment Profile Details - If [ Q_SP_Status <> Invalid ] |  |  |  |  |  |  |  |  |
|  |  | M_SP_Version (k) |  |  |  | If [ Q_SP_Status <> Invalid ] Identifier of the segment profile version. | 16 bits | 1 byte for the major version and 1 byte for the minor one. |
|  |  | L_SP (k) |  |  |  | If [ Q_SP_Status <> Invalid ] Length of the segment of railway covered by the SP. | 24 bits | Binary to numeric (in centimetres) Minimum Value: 1 |
|  |  | D_EoA_Offset (k) |  |  |  | If [ Q_SP_Status <> Invalid ] Distance to stop the train in rear of the EoA. | 24 bits | Binary to numeric (in centimetres) |
|  |  | Q_UTC_Offset (k) |  |  |  | If [ Q_SP_Status <> Invalid ] Offset to add to the UTC time in order to calculate the local time. | 7 bits | Binary to numeric Unsigned value Resolution: 15 min 0 = UTC - 14:00 56 = UTC ± 0 112 = UTC + 14:00 113 - 127 = Spare |
|  |  | M_SP_Altitude (k) |  |  |  | If [ Q_SP_Status <> Invalid ] Altitude at the beginning of the SP. Considering ETRS89 as reference. Note: ETRS89 is the EU-recommended frame of reference for geodata for Europe | 20 bits | Binary to numeric (in centimetres) starting at -1000m |
|  |  | Q_SP_OperationalContext(k) |  |  |  | If [ Q_SP_Status <> Invalid ] Grade of Automation depending on the fitment of the trackside. It represents the maximum GoA allowed. | 3 bits | 0 = GoA0 1 = GoA1 2 = GoA2 3 = GoA3 4 = GoA4 [5-7] = Spare |
|  |  | Q_ATOTS_Contact_Info_Dir (k) |  |  |  | If [ Q_SP_Status <> Invalid ] Qualifier indicating whether contact information of another ATO-TS valid for nominal travelling direction of the SP, valid for reverse travelling direction of the SP or no contact information follows. | 2 bits | 0 = No Contact info follows 1 = ATO-TS contact info for nominal direction follows 2 = ATO-TS contact info for reverse direction follows 3 = Spare |
| ATO-TS Contact Information - If [ Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for nominal direction follows or Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for reverse direction follows ] |  |  |  |  |  |  |  |  |
|  |  | NID_C (k) |  |  |  | If [ Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for nominal direction follows or Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for reverse direction follows ] Identity of the ATO-TS’s country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  | NID_ATOTS (k) |  |  |  | If [ Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for nominal direction follows or Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for reverse direction follows ] Identifier of the adjacent ATO-TS. The value of this variable is assigned to be unique, see clause 6.2.1.10 | 14 bits | Binary to numeric |
|  |  | NID_C (k) |  |  |  | If [ Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for nominal direction follows or Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for reverse direction follows ] Identity of the SP’s country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  | NID_SP (k) |  |  |  | If [ Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for nominal direction follows or Q_ATOTS_Contact_Info_Dir == ATO-TS contact info for reverse direction follows ] SP identity of the first segment in the adjacent ATO-TS area. | 32 bits | Binary to numeric |
| Static Speed Profile Start |  |  |  |  |  |  |  |  |
|  |  | V_STATIC (k) |  |  |  | Basic Static Speed Profile speed at the beginning of the Segment Profile. The variable used is comparable to V_STATIC, as defined in [Ref 2], Section 7.5.1.171. | 7 bits | [0..120] x 5 km/h <=> [0..600] km/h; [121-127] = Spare |
|  |  | Q_FRONT (k) |  |  |  | see [Ref 2], Section 7.5.1.109 This is applicable only for step up. | 1 bits | see [Ref 2], Section 7.5.1.109 |
| Specific SSP |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Specific Static Speed Profiles. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_DIFF (k,l) |  |  | See [Ref 2], Section 7.5.1.102.1. | 2 bits | See [Ref 2], Section 7.5.1.102.1. |
| Cant Deficiency - If [ Q_DIFF == Cant Deficiency specific category ] |  |  |  |  |  |  |  |  |
|  |  |  | NC_CDDIFF (k,l) |  |  | If [ Q_DIFF == Cant Deficiency specific category ] See [Ref 2], Section 7.5.1.82.1 | 4 bits | See [Ref 2], Section 7.5.1.82.1 |
| Other Specific SSP - If [ Q_DIFF == Other specific category, replaces the Cant Deficiency SSP or Q_DIFF == Other specific category, does not replace the Cant Deficiency SSP ] |  |  |  |  |  |  |  |  |
|  |  |  | NC_DIFF (k,l) |  |  | If [ Q_DIFF == Other specific category, replaces the Cant Deficiency SSP or Q_DIFF == Other specific category, does not replace the Cant Deficiency SSP ] See [Ref 2], Section 7.5.1.83. | 4 bits | See [Ref 2], Section 7.5.1.83. |
|  |  |  | V_DIFF (k,l) |  |  | See [Ref 2], Section 7.5.1.156 | 7 bits | See [Ref 2], Section 7.5.1.156 |
| Static Speed Profile Change |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Static Speed Profiles changes. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | D_Location (k,l) |  |  | Location of the Static Speed Profile change relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | V_STATIC (k,l) |  |  | Basic Static Speed Profile speed. The variable used is comparable to V_STATIC, as defined in [Ref 2], Section 7.5.1.171. | 7 bits | [0..120] x 5 km/h <=> [0..600] km/h; [121-127] = Spare |
|  |  |  | Q_FRONT (k,l) |  |  | See [Ref 2], Section 7.5.1.109 This is applicable only for step up. | 1 bits | see [Ref 2], Section 7.5.1.109 |
| Specific SSP Change |  |  |  |  |  |  |  |  |
|  |  |  | N_ITER (k,l) |  |  | Number of iterations of Specific Static Speed Profiles. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  |  | Q_DIFF (k,l,m) |  | See [Ref 2], Section 7.5.1.102.1. | 2 bits | See [Ref 2], Section 7.5.1.102.1. |
| Cant Deficiency Change - If [ Q_DIFF == Cant Deficiency specific category ] |  |  |  |  |  |  |  |  |
|  |  |  |  | NC_CDDIFF (k,l,m) |  | If [ Q_DIFF == Cant Deficiency specific category ] See [Ref 2], Section 7.5.1.82.1 | 4 bits | See [Ref 2], Section 7.5.1.82.1 |
| Other Specific SSP Change - If [ Q_DIFF == Other specific category, replaces the Cant Deficiency SSP or Q_DIFF == Other specific category, does not replace the Cant Deficiency SSP ] |  |  |  |  |  |  |  |  |
|  |  |  |  | NC_DIFF (k,l,m) |  | If [ Q_DIFF == Other specific category, replaces the Cant Deficiency SSP or Q_DIFF == Other specific category, does not replace the Cant Deficiency SSP ] See [Ref 2], Section 7.5.1.83. | 4 bits | See [Ref 2], Section 7.5.1.83. |
|  |  |  |  | V_DIFF (k,l,m) |  | See [Ref 2], Section 7.5.1.156 | 7 bits | See [Ref 2], Section 7.5.1.156 |
| Gradient Start |  |  |  |  |  |  |  |  |
|  |  | G_New_Gradient (k) |  |  |  | Value of the new gradient at the beginning of the Segment Profile. The variable used is comparable to G_A, as defined in [Ref 2], Section 7.4.2.6. | 10 bits | Binary to numeric (0-1023) with resolution 0.1‰ |
|  |  | Q_GDIR (k) |  |  |  | see [Ref 2], Section 7.5.1.110 | 1 bits | see [Ref 2], Section 7.5.1.110 |
| Gradients Change |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of gradient changes. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | D_Location (k,l) |  |  | Location of the gradient change relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | G_New_Gradient (k,l) |  |  | Value of the new gradient. The variable used is comparable to G_A, as defined in [Ref 2], Section 7.4.2.6. | 10 bits | Binary to numeric (0-1023) with resolution 0.1‰ |
|  |  |  | Q_GDIR (k,l) |  |  | see [Ref 2], Section 7.5.1.110 | 1 bits | see [Ref 2], Section 7.5.1.110 |
| Curve Start |  |  |  |  |  |  |  |  |
|  |  | Q_Radius_Category (k) |  |  |  | Curve category at the beginning of the Segment Profile. | 5 bits | 0 = R>7000m 1 = 7000m≥R>4500m 2 = 4500m≥R>2800m 3 = 2800m≥R>2000m 4 = 2000m≥R>1500m 5 = 1500m≥R>1250m 6 = 1250m≥R>1075m 7 = 1075m≥R>925m 8 = 925m≥R>825m 9 = 825m≥R>725m 10 = 725m≥R>625m 11 = 625m≥R>525m 12 = 525m≥R>475m 13 = 475m≥R>425m 14 = 425m≥R>375m 15 = 375m≥R>325m 16 = 325m≥R>300m 17 = 300m≥R>275m 18 = 275m≥R>250m 19 = 250m≥R>225m 20 = 225m≥R>200m 21 = 200m≥R>175m 22 = 175≥R>150m 23 = = R≤150m [24-31] = Spare |
| Curves Change |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of curve changes. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | D_Location (k,l) |  |  | Location of the curve change relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | Q_Radius_Category (k,l) |  |  | Curve category. | 5 bits | 0 = R>7000m 1 = 7000m≥R>4500m 2 = 4500m≥R>2800m 3 = 2800m≥R>2000m 4 = 2000m≥R>1500m 5 = 1500m≥R>1250m 6 = 1250m≥R>1075m 7 = 1075m≥R>925m 8 = 925m≥R>825m 9 = 825m≥R>725m 10 = 725m≥R>625m 11 = 625m≥R>525m 12 = 525m≥R>475m 13 = 475m≥R>425m 14 = 425m≥R>375m 15 = 375m≥R>325m 16 = 325m≥R>300m 17 = 300m≥R>275m 18 = 275m≥R>250m 19 = 250m≥R>225m 20 = 225m≥R>200m 21 = 200m≥R>175m 22 = 175≥R>150m 23 = = R≤150m [24-31] = Spare |
| Power Voltage Start |  |  |  |  |  |  |  |  |
|  |  | M_VOLTAGE (k) |  |  |  | See [Ref 2], Section 7.5.1.78 | 4 bits | See [Ref 2], Section 7.5.1.78 |
|  |  | NID_CTRACTION (k) |  |  |  | If [ M_VOLTAGE <> Line not fitted with any traction system ] See [Ref 2], Section 7.5.1.86.1 | 10 bits | See [Ref 2], Section 7.5.1.86.1 |
| Power Voltage Change |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of voltage changes. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | D_Location (k,l) |  |  | Location of the voltage change relatively to the beginning of the SP | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | M_VOLTAGE (k,l) |  |  | See [Ref 2], Section 7.5.1.78 | 4 bits | See [Ref 2], Section 7.5.1.78 |
|  |  |  | NID_CTRACTION (k,l) |  |  | If [ M_VOLTAGE <> Line not fitted with any traction system ] See [Ref 2], Section 7.5.1.86.1 | 10 bits | See [Ref 2], Section 7.5.1.86.1 |
| Current Limitation Start |  |  |  |  |  |  |  |  |
|  |  | M_CURRENT (k) |  |  |  | See [Ref 2], Section 7.5.1.62.1 Note: a powerless section is indicated as a current limit “0” (zero) | 10 bits | See [Ref 2], Section 7.5.1.62.1 |
| Current Limitation Change |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of allowed current consumption changes. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | D_Location (k,l) |  |  | Location of the allowed current consumption change relatively to the beginning of the SP | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | M_CURRENT (k,l) |  |  | Current Consumption Limit See [Ref 2], Section 7.5.1.62.1 Note: a powerless section is indicated as a current limit “0” (zero) | 10 bits | See [Ref 2], Section 7.5.1.62.1 |
| Balise Group |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of balise groups. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_NEWNID_C (k,l) |  |  | Qualifier to indicate if NID_C of this balise group is following or the NID_C of the SP is valid for this balise group. | 1 bits | 0 = Use NID_C of SP 1 = NID_C to be used follows |
|  |  |  | NID_C (k,l) |  |  | If [ T_Q_NEWNID_C == NID_C to be used follows ] Identity of the BG's country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  |  | NID_BG (k,l) |  |  | See [Ref 2], Section 7.5.1.85. | 14 bits | See [Ref 2], Section 7.5.1.85. |
| Balises |  |  |  |  |  |  |  |  |
|  |  |  | N_ITER_BG (k,l) |  |  | Number of iterations of balises in the balise group, which are located on the kth SP Note: the number of balises belonging to a balise group and located on an SP may be lower than the total number of balises of the balise group, in case this latter is split over two SPs. | 4 bits | Binary to numeric |
|  |  |  |  | N_PIG (k,l,m) |  | See [Ref 2], Section 7, 5.1.81 | 3 bits | See [Ref 2], Section 7, 5.1.81 |
|  |  |  |  | D_Location (k,l,m) |  | Location of the balise relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Timing Points |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Timing Points. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | NID_TP (k,l) |  |  | TP identity. The NID_TP is unique within a NID_C. | 32 bits | Binary to numeric. |
|  |  |  | D_Location (k,l) |  |  | Location of the Timing Point relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | Q_Stop_Location_Tolerance (k,l) |  |  | Required stopping tolerance to use when the TP is a Stopping Point. | 5 bits | 0 = 10cm 1 = 20cm 2 = 30cm 3 = 40cm 4 = 50cm 5 = 1m 6 = 1,5m 7 = 2m 8 = 2,5m 9 = 3m 10 = 5m 11 = 7,5m 12 = 10m 13 = 15m 14 = 20m 15 = 25m 16 = 30m 17 = 50m 18 = 75m 19 = 100m [20-30] = Spare 31 = No requirement |
|  |  |  | Q_STP_Reached (k,l) |  |  | Distance from a Stopping Point to consider it as reached. | 5 bits | 0 = 10cm 1 = 20cm 2 = 30cm 3 = 40cm 4 = 50cm 5 = 1m 6 = 1,5m 7 = 2m 8 = 2,5m 9 = 3m 10 = 5m 11 = 7,5m 12 = 10m 13 = 15m 14 = 20m 15 = 25m 16 = 30m 17 = 50m 18 = 75m 19 = 100m [20-30] = Spare 31 = No requirement |
| Timing Point Name |  |  |  |  |  |  |  |  |
|  |  |  | L_TEXT (k,l) |  |  | See [Ref 4], Section 8.1.11. | 8 bits | See [Ref 4], Section 8.1.11. |
|  |  |  |  | X_TEXT (k,l,m) |  | Name of the TP. | 8 bits | See [Ref 4], Section 8.1.120. |
| Platform Area |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Platform Areas. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Platform Area starts, ends, starts and ends or covers the whole concerning Segment Profile | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the platform start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the platform end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Tunnel |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of tunnels. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Tunnel starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | Q_Tunnel_Category (k,l) |  |  | Category of the Tunnel. | 2 bits | 0 = Single track tunnel 1 = Double track tunnel 2 = Wide-cross section tunnel 3 = Spare |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the tunnel start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the tunnel end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Axle Load Speed Profile |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Axle Load Speed Profiles. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Axle Load Speed Profile starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | M_AXLELOADCAT (k,l) |  |  | See [Ref 2], Section 7.5.1.62 | 7 bits | See [Ref 2], Section 7.5.1.62 |
|  |  |  | V_New_Speed_Level (k,l) |  |  | Speed restriction to be applied if the axle load of the train ≥ M_AXLELOADCAT (k,v) | 7 bits | [0..120] x 5 km/h <=> [0..600] km/h; [121-127] = Spare |
|  |  |  | Q_FRONT (k,l) |  |  | See [Ref 2], Section 7.5.1.109 This is applicable only for step up. | 1 bits | see [Ref 2], Section 7.5.1.109 |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Axle Load Speed Profile start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Axle Load Speed Profile end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Unprotected Level Crossing Stop |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of stopping locations for unprotected level crossings. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | D_UnprotectedLx_Stop_Nominal (k,l) |  |  | Location of the stop in rear of an unprotected level crossing if the valid travelling direction of the SP is nominal. Undefined if for this travelling direction the location is not inside this SP. | 24 bits | Binary to numeric (in centimetres) 16777215 = Undefined Location |
|  |  |  | D_UnprotectedLx_Stop_Reverse (k,l) |  |  | Location of the stop in rear of an unprotected level crossing if the valid travelling direction of the SP is reverse. Undefined if for this travelling direction the location is not inside this SP. | 24 bits | Binary to numeric (in centimetres) 16777215 = Undefined Location |
| Permitted Braking Distance |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Permitted Braking Distance areas. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Permitted Braking Distance area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_Permitted_Braking_Distance (k,l) |  |  | Permitted Braking Distance value. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | Q_PBD_SBEB (k,l) |  |  | Whether the permitted braking distance is to be achieved with the Service Brake or Emergency Brake. | 1 bits | 0 = Service Brake 1 = Emergency Brake |
|  |  |  | G_PBD (k,l) |  |  | A single gradient value applicable for the calculation | 8 bits | Binary to numeric (0-255) with resolution 1‰ |
|  |  |  | Q_GDIR_PBD (k,l) |  |  | Direction of the gradient. | 1 bits | 0 = Downhill 1 = Uphill |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Permitted Braking Distance area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Permitted Braking Distance area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Switch Off Regenerative Brake |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Switch off Regenerative Brake areas. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Switch off Regenerative Brake area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Switch off Regenerative Brake area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Switch off Regenerative Brake area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Switch Off Eddy Current Brake |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Switch off eddy current brake for service brake areas. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Switch off eddy current brake for service brake area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Switch off eddy current brake for service brake area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Switch off eddy current brake for service brake area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Switch Off Eddy Current Emergency Brake |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Switch off eddy current brake for emergency brake areas. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Switch off eddy current brake for emergency brake area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Switch off eddy current brake for emergency brake area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Switch off eddy current brake for emergency brake area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Switch Off Magnetic Shoe Brake |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of Switch off Magnetic Shoe Brake areas. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Switch off Magnetic Shoe Brake area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = StartsEnds 3 = WholeSP |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Switch off Magnetic Shoe Brake area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Switch off Magnetic Shoe Brake area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Area with Dynamic Brake force Inhibition |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of areas with Dynamic Brake force inhibition. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Dynamic Brake Inhibition Area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = forceStartsEnds 3 = WholeSP |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Dynamic Brake Force Inhibition Area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Dynamic Brake Force Inhibition Area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
| Area with limited Dynamic Brake force |  |  |  |  |  |  |  |  |
|  |  | N_ITER (k) |  |  |  | Number of iterations of areas with limited Dynamic Brake force. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  |  | Q_Range (k,l) |  |  | Specifies if the Dynamic Brake Limitation Area starts, ends, starts and ends or covers the whole concerning Segment Profile. | 2 bits | 0 = Starts 1 = Ends 2 = forceStartsEnds 3 = WholeSP |
|  |  |  | M_Dynamic_Brake_Force_Limit (k,l) |  |  | Maximum Brake Force [kN] to be applied by the Dynamic Brake | 16 bits | Range: 0 ... 3000 kN, resolution 1 kN, Special values: 3001 ... MAXNUM - 1: spare MAXNUM: Unknown |
|  |  |  | D_Start_Location (k,l) |  |  | If [ Q_Range == Starts or Q_Range == StartsEnds ] Location of the Dynamic Brake Force Limit Area start relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |
|  |  |  | D_End_Location (k,l) |  |  | If [ Q_Range == Ends or Q_Range == StartsEnds ] Location of the Dynamic Brake Force Limit Area end relatively to the beginning of the SP. | 24 bits | Binary to numeric (in centimetres) |

Table 11 Segment Profile Packet structure

#### If the status of the Segment Profile is invalid, the Segment Profile Details shall finish after Q_SP_Status (Item 5).

#### Segment Profile Location Dependent Data Types

##### All location dependent data types of a Segment Profile can be clustered into the following categories, which are explained in the consecutive chapters:

- ATO Position Types

- ATO Profile Types

- ATO Area Types

##### Table 14 assigns the category to the location dependent ATO data types.

| Location Dependent Data | ATO Type |
| --- | --- |
| Static Speed Profile | Profile |
| Gradients | Profile |
| Curves | Profile |
| Power Voltage | Profile |
| Current Limitation | Profile |
| Balises | Position |
| Timing Points | Position |
| Platform Areas | Area |
| Tunnels | Area |
| Axle Load Speed Profiles | Area |
| Unprotected LX | Position |
| Permitted Braking Distances | Area |
| Switch off regenerative brake areas | Area |
| Switch off eddy current brake for SB | Area |
| Switch off eddy current brake for EB | Area |
| Switch off magnetic shoe brake | Area |
| Inhibition of the Dynamic Brake | Area |
| Dynamic Brake Force Limit | Area |

Table 12: Type Classification for Segment Profile Data

#### ATO Position Types

##### ATO Position Type elements are described by a distance relative to a reference point only and do not have any extension. Consequently, the location dependent data assigned to the Segment Profile is applicable at this position only.

##### At a specific position, only one element of the same position type shall be specified.

##### The data shall be sorted in ascending order of position.

#### ATO Profile Types

##### An ATO Profile Type always starts at the Segment Profile’s start point and is applicable till (excluding) the next discontinuity given in the profile, if any.

##### For each ATO Profile Type, the element at the Segment Profile’s origin shall be given.

##### The position of any discontinuity shall not be located at the Segment Profile’s start point. Discontinuities are optional.

##### At a specific position, only one element of the same ATO Profile Type shall be specified.

##### The data shall be sorted in ascending order of position.

#### ATO Area Types

##### An ATO Area Type is defined by its start and end location and can cover multiple Segment Profiles. Only in case the area extremities are located within this particular Segment Profile they shall be given in this Segment Profile.

##### Data belonging to the same type/category (if applicable, e.g. M_AXLELOADCAT) shall be sorted in ascending order of their start offset and shall neither overlap nor be nested.

##### If the start offset is not given, it is assumed as zero.

##### The end offset, if given, shall be unequal to the start offset of this particular type.

#### Speed values

##### If at the same position multiple speed values of different categories/types (beside the static speed profile) are defined, they shall be sorted according to their category criterion. The sorting depends on the category/type:

- NC_CDDIFF: Ascending

- NC_DIFF: Ascending

- M_AXLELOADCAT: Ascending

### Status Report Packet (STR)

#### The section details the structure of a Status Report Packet sent by the ATO-OB to the ATO-TS.

#### Status Report Packet composition:

| Packet Number |  |  | 8 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  |  | Description | Length | Resolution/Formula |
|  | ATO Header |  |  | see chapter 7.2 |  | - |
| Status Report Details |  |  |  |  |  |  |
|  | M_ATO_State |  |  | The current ATO State in use. | 4 bits | 0 = Unknown 1 = CO 2 = NA 3 = AV 4 = RE 5 = EG 6 = DE 7 = FA [8-15] = Spare |
|  | Q_STR_Indicators |  |  | Bitset with the indicators state. | 16 bits | bit0 = JP SP Consistency Error bit1 = Routing Error bit2 = Next Stopping Point Skip bit3 = Low adhesion reported by the driver bit4 = Operational conditions fulfilment bit5 = Train is moving bit6 = Unable to stop at the next Stopping Point bit7 = Slip/slide reported by TCMS/Train bit8 = JP completed [bit9-bit15] = Spare |
|  | V_TRAIN_ATO |  |  | Current speed of the train when the STR is sent. | 10 bits | Binary to numeric. Resolution: 1 km/h |
|  | L_TRAIN |  |  | See [Ref 2], Section 7.5.1.56. | 12 bits | See [Ref 2], Section 7.5.1.56. |
|  | DRIVER_ID |  |  | See [Ref 6], Section 4.2.3.7. | 128 bits | See [Ref 6], Section 4.2.3.7. |
| STR Sending Location |  |  |  |  |  |  |
|  | NID_C |  |  | Identity of the SP's country or region. Not relevant if D_Sending_Position = Undefined Location. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  | NID_SP |  |  | SP identity. Not relevant if D_Sending_Position = Undefined Location. | 32 bits | Binary to numeric |
|  | D_Sending_Position |  |  | Position of the estimated front end of the train at the moment the STR is sent (relatively from the beginning of the given SP). | 24 bits | Binary to numeric (in centimetres) 16777215 = Undefined Location |
| Previous TP Information |  |  |  |  |  |  |
|  | NID_C |  |  | Identity of the previous TP's country or region. Not relevant if NID_TP is undefined. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  | NID_TP |  |  | Previous TP identity. The NID_TP is unique within a NID_C. | 32 bits | Binary to numeric. 4294967295 for undefined value. |
|  | Q_Pass_Stop_Depart |  |  | Qualifier to indicate if train has stopped at the TP, has departed from the TP or has passed the TP. | 2 bits | 0 = Train passed the TP 1 = Train stopped at the TP 2 = Train departed from the TP 3 = Undefined |
| Accurate Stopping - If [ Q_Pass_Stop_Depart == Train stopped at the TP ] |  |  |  |  |  |  |
|  | Q_Accurate_Stopping |  |  | If [ Q_Pass_Stop_Depart == Train stopped at the TP ] This qualifier specifies if the train has stopped accurately or not at the Operational Stopping Point. | 2 bits | 0 = Undershoot 1 = Accurate 2 = Overshoot 3 = Spare |
| Timing Point Estimation |  |  |  |  |  |  |
|  | N_ITER |  |  | Number of iterations of TPs information. If N_ITER is 0 then no data set is following. | 5 bits | See [Ref 2], Section 7.5.1.80 |
|  |  | NID_C (k) |  | Identifier of the next TP's country or region. | 10 bits | See [Ref 2], Section 7.5.1.86. |
|  |  | NID_TP (k) |  | Next TP identity. The NID_TP is unique within a NID_C. | 32 bits | Binary to numeric. |
|  |  | T_Arrival_Date (k) |  | Date to arrive at the TP. | 15 bits | Date of a timestamp in UTC. The variable represents the number of days from the 1st January 2010. Values from 0 (01/01/2010) to 32767 (18/09/2099) |
|  |  | T_Arrival_Seconds (k) |  | Estimated time in seconds to arrive at the TP. | 17 bits | Seconds of a timestamp in UTC. The variable represents the number of seconds from the beginning of the day. Values from 0 (00:00:00) to 86400 (24:00:00) |

Table 13 Status Report Packet structure

### Status Report Acknowledgement Packet (STRAck)

#### The section details the structure of a Status Report Acknowledgement Packet sent by the ATO-TS to the ATO-OB.

#### Status Report Acknowledgement Packet composition:

| Packet Number |  | 9 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |
| Status Report Acknowledgement Details |  |  |  |  |  |
|  | T_STR_Reference_Timestamp_Date |  | Date of the timestamp of the Status Report Packet (see 7.2 ATO Header) for which the acknowledgement is provided. | 15 bits | Date of a timestamp in UTC. The variable represents the number of days from the 1st January 2010. Values from 0 (01/01/2010) to 32767 (18/09/2099) |
|  | T_STR_Reference_Timestamp_Seconds |  | Seconds of the timestamp of the Status Report Packet in UTC (see 7.2 ATO Header) for which the acknowledgement is provided. | 17 bits | Seconds of a timestamp in UTC. The variable represents the number of seconds from the beginning of the day. Values from 0 (00:00:00) to 86399 (23:59:59) |
|  | N_STR_Reference_Packet_Counter |  | N_Packet_Counter of the Status Report (see 7.2 ATO Header) for which the acknowledgement is provided. | 8 bits | Binary to numeric |

Table 14 Status Report Acknowledgement Packet structure

### Session Termination Request Packet (SESSTermReq)

#### The section details the structure of a Session Termination Request Packet sent by the ATO-TS to the ATO-OB.

#### Session Termination Request Packet composition:

| Packet Number |  | 10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |

Table 15 Session Termination Request Packet structure

### Session Termination Packet (SESSTerm)

#### The section details the structure of a Session Termination Packet sent by the ATO-OB to the ATO-TS.

#### Session Termination Packet composition:

| Packet Number |  | 11 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Variable Name |  | Description | Length | Resolution/Formula |
|  | ATO Header |  | see chapter 7.2 |  | - |
| Session Termination Details |  |  |  |  |  |
|  | Q_Termination_Reason |  | Qualifier indicating the reason for the communication session termination. | 4bits | 0 = End of Journey reached 1 = ATO-TS Termination Request 2 = Last SP left 3 = Cab inactive 4 = TRN or train length not valid 5 = ETCS-OB in NL 6 = ETCS-OB in SH 7 = ATO TS Handover cancelled 8 - 15 = Spare |

Table 16 Session Termination Packet structure

# Appendix

## A.1. 	Example Encoded Handshake Request Packet

### A.1.1. 	The following example shows the encoding of a Handshake Request Packet.

Figure 1 Encoded Handshake Request Packet

The hex string for the Handshake Request Packet example is as follows:

- 0x00100210FF870A4F22F2A8BB01080800