# Conformance test cases BL4R3

Version 2 (publication March 2026)

# Changes/corrections based on feedback from IMs and industry

Clarification of changes compared to test cases BL4R3 - Version 1 (publication June 2025)

Document History and Distribution

Revision History

| Date | Version | Comments |
| --- | --- | --- |
| March 2026 | V.01.00 | Initial version |

Table of contents

1	General	5

1.1	Placeholders <parameter name> in test cases	5

1.2	Conditions for successful test case result	5

1.3	Fallback mode	5

1.4	Update parameter DIR on test case steps	5

1.5	Correction of config parameter names	6

1.6	Populating missing values on parameter Instance	7

1.7	Enhancement on field Formalisation on steps PDI_TELEGRAM	7

2	EfeS Generic	8

3	SubS Light Signal	9

3.1	General note	9

3.2	Changes on particular test cases	9

3.2.1	TC_Eu.LS.5241.01	9

3.2.2	TC_Eu.LS.5128.01	10

3.2.3	TC_Eu.LS.5097.01	10

3.2.4	TC_Eu.LS.5044.01	10

3.2.5	TC_Eu.LS.5157.01	10

3.2.6	TC_Eu.LS.5261.01	10

3.2.7	TC_Eu.LS.5138.01	11

3.2.8	TC_Eu.LS.5341.01	11

3.2.9	TC_Eu.LS.5322.01	11

3.2.10	TC_Eu.LS.4975.01	11

3.2.11	TC_Eu.LS.5147.01	12

3.2.12	TC_Eu.LS.5207.01	12

3.2.13	TC_Eu.LS.5261.01	12

3.2.14	TC_Eu.LS.7953.01	12

3.2.15	TC_Eu.LS.5293.01	12

3.2.16	TC_Eu.LS.7777.01	13

3.2.17	Change Option to Option LS4 and Option LS5 - multiple test cases	13

4	SubS Generic IO	14

4.1	Changes of timing requirements - multiple test cases	14

4.2	Activation delay - multiple test cases	14

4.3	Other changes on particular test cases	15

4.3.1	TC_Eu.IO.495.01	15

4.3.2	TC_Eu.IO.1718	15

4.3.3	TC_Eu.IO.1730	15

4.3.4	TC_Eu.IO.7688, TC_Eu.IO.7689	15

5	SubS Level Crossing	16

6	SubS TDS	17

6.1	Changes on particular test cases	17

6.1.1	TC_Eu.TDS.1954.01	17

6.1.2	TC_Eu.TDS.5979.01	17

6.1.3	TC_Eu.TDS.5983.01	17

6.1.4	TC_Eu.TDS.6001.01	17

6.1.5	TC_Eu.TDS.5987.01	17

6.1.6	TC_Eu.TDS.6014.01	17

7	SubS Point	18

7.1	Changes on particular test cases	18

7.1.1	TC_Eu.P.6761.01	18

7.1.2	TC_Eu.P.3093.01	18

7.1.3	TC_Eu.P.1469.01  and TC_Eu.P.5371.01	18

7.1.4	TC_Eu.P.7098.01	18

8	Other	18

# General

Based on feedback from TACS group corrections were applied. It was agreed that after corrections all test cases will be exported again and all test cases which are part of export have Version parameter set to value 2.0 (regardless if TC needed correction or not).

## Placeholders <parameter name> in test cases

In test cases some of parameters are specified within angle brackets <>. Examples:

<P_INTERLOCKING_IDENTIFIER>

<P_SUBSYSTEM_IDENTIFIER>

<P_CHECKSUM_LENGTH>

Values of this parameters shall be populated based on configuration or test bench and configuration of device under test.

## Conditions for successful test case result

Test case can be treated as “successful” if following criteria are met:

- Tested device shall respond as described in test steps, in defined order and with defined timing,

- PDI telegrams are defined, control outputs as defined,

- Postconditions met, shall be stable for longer time (eg. 5 seconds after finishing last test step)

- no other telegrams or control outputs shall arrive to test bench during the test steps

## Fallback mode

Due to fact that Fallback mode is difficult to be tested (testing of this is difficult because introduction of internal error IE.01 can be impossible by some implementations) test cases related to Fallback mode are tagged/labelled with parameter “Options” with value “Option Fallback mode”.

Impacted test cases:

- TC_Eu.IO.506.01

- TC_Eu.LC.1013.01

- TC_Eu.LS.5573.01

- TC_Eu.P.1252.01

- TC_Eu.TDS.6874.01

## Update parameter DIR on test case steps

Parameter DIR was updated on multiple test steps (PDI telegram, PHYSICAL_INTERFACE where parameter was not populated)

Parameter DIR was set to L on all test case steps LOGICAL_STATE

## Correction of config parameter names

Configuration parameter IO.DISTURBANCE_CONFIG_ALL_OFF renamed to P_CON_ DISTURBANCE_CONFIG_ALL_OFF – impacted test cases:

- TC_Eu.IO.1611.01

- TC_Eu.IO.1788.01

- TC_Eu.IO.497.01

- TC_Eu.IO.506.01

- TC_Eu.IO.508.01

- TC_Eu.IO.509.01

- TC_Eu.IO.510.01

Configuration parameter CON_DOWNGRADE_MOST_RESTRICT renamed to LS.CON_DOWNGRADE_MOST_RESTRICT, impacted test cases:

- TC_Eu.LS.5147.01

Config parameters names for LS test cases got prefix LS.CON_ to follow naming convention. Impacted test cases:

- TC_Eu.LS.4975.01

- TC_Eu.LS.4996.01

- TC_Eu.LS.5006.01

- TC_Eu.LS.5034.01

- TC_Eu.LS.5060.01

- TC_Eu.LS.5069.01

- TC_Eu.LS.5079.01

- TC_Eu.LS.5097.01

- TC_Eu.LS.5128.01

- TC_Eu.LS.5138.01

- TC_Eu.LS.5207.01

- TC_Eu.LS.5241.01

- TC_Eu.LS.5304.01

- TC_Eu.LS.5322.01

- TC_Eu.LS.5508.01

- TC_Eu.LS.7951.01

- TC_Eu.LS.7952.01

- TC_Eu.LS.7953.01

Configuration parameter in test cases for Level Crossing LC4out.FAILURE_STATUS_AFTER_PRESENT_FAILURE was renamed to

LC.CON_ FAILURE_STATUS_AFTER_PRESENT_FAILURE (alignment on naming convention)

Impacted test cases:

- TC_Eu.LC.978.01

- TC_Eu.LC.978.02

- TC_Eu.LC.1000.01

- TC_Eu.LC.1000.02

Configuration parameters in test cases for Level Crossing were renamed LC4in.xxx -> LC.CON_xxx (alignment of parameters naming convention).

- LC4in.HAS_BARRIER_MOVEMENT

- LC4in.HAS_BARRIER_POSITION

- LC4in.HAS_HARDWARE_STATUS

- LC4in.HAS_ROAD_LIGHT_STATUS

Parameters were renamed to:

- LC.CON_HAS_BARRIER_MOVEMENT

- LC.CON_HAS_BARRIER_POSITION

- LC.CON_HAS_HARDWARE_STATUS

- LC.CON_HAS_ROAD_LIGHT_STATUS

Impacted test cases: TC_Eu.LC.191.01.

## Populating missing values on parameter Instance

Populated values on field Instance where value was missing (on preconditions, postconditions, config, steps), several test cases.

## Enhancement on field Formalisation on steps PDI_TELEGRAM

Texts in field formalisation are now machine generated from PDI_TELEGRAM step data model and contains more readable text explaining both byte position meaning and byte value meaning for each byte of PDI telegram – examples:

COMMAND; PDI[00..00] = 0x30 (Protocol Type: 0x30=Electronic Interlocking - Light Signal); PDI[01..02] = 0x0001 (MessageType: 0x0001 = Command: Indicate Signal Aspect); PDI[03..22] = <P_ILS_ID_TEC> (Sender Identifier (20bytes): <P_ILS_ID_TEC>); PDI[23..42] = <P_LS_ID_OP> (Receiver Identifier (20 Bytes): <P_LS_ID_OP>); PDI[43..43] = 0x10 (Code for basic aspect types: 0x10 = Limited speed / Expect clear 35); PDI[44..44] = 0xFF (Code for extension of basic aspect types: 0xFF = Intended dark / Not equipped / No information); PDI[45..45] = 0x08 (Speed indicators: 0x08 = Speed indication (80 km/h)); PDI[46..46] = 0xFF (Speed indicator announcements: 0xFF = Speed announcement dark / Not equipped / No information); PDI[47..47] = 0xFF (Direction indicators: 0xFF = Direction indication dark / Not equipped / No information); PDI[48..48] = 0xFF (Direction indicator announcements: 0xFF = Direction announcement dark / Not equipped / No information); PDI[49..49] = 0xFF (Downgrade information: 0xFF = Downgrade information not applicable); PDI[50..50] = 0xFF (Route information: 0xFF = Route information not applicable); PDI[51..51] = 0x0F (Signal Aspect intentionally dark: 0x0F = The commanded Signal Aspect or the related downgraded Aspect shall be indicated dark); PDI[52..60] = 0xFEFEFEFEFEFEFEFEFE (National specified: 0xFEFEFEFEFEFEFEFEFE);

MESSAGE; PDI[00..00] = 0x20 (Protocol Type: 0x20=Electronic Interlocking - Train Detection System); PDI[01..02] = 0x0007 (MessageType: 0x0007 = Message: TVPS Occupancy Status); PDI[03..22] = <P_TDS_ID_OP> (Sender Identifier (20bytes): <P_TDS_ID_OP>); PDI[23..42] = <P_ILS_ID_TEC> (Receiver Identifier (20 Bytes): <P_ILS_ID_TEC>); PDI[43..43] = 0x02 (Occupancy Status: 0x02 = TVPS is in state occupied); PDI[44..44] = 0x01 (Ability to be forced to clear: 0x01 = TVPS is not able to be forced to clear); PDI[45..46] = 0xFFFF (Filling Level: 0xFFFF = Filling Level is not applicable); PDI[47..47] = 0x01 (POM Status: 0x01 = Power supply OK); PDI[48..48] = 0xFF (Disturbance Status: 0xFF = Disturbance status is not applicable); PDI[49..49] = 0x01 (Change Trigger: 0x01 = Passing detected);

## Parameter Func. Pkg. in JSON formalisation

Parameter Func. Pkg. was promoted to “top-level” parameter from being before nested in object "legacy_xlsx_additional_data".

Texts in field formalisation are now machine generated from PDI_TELEGRAM step data model and contains more readable text explaining both byte position meaning and byte value meaning for each byte of PDI telegram – examples:

# EfeS Generic

Test cases:

- TC_Eu.Gen-SCI.47.01

- TC_Eu.Gen-SCI.48.01

- TC_Eu.Gen-SCI.49.01

- TC_Eu.Gen-SCI.51.01

- TC_Eu.Gen-SCI.52.01

were removed from test case library since this test cases relates to testing EULYNX adjacent systems and not EfeS.

# SubS Light Signal

## General note

Allowed combinations of parameters on “Train driver” interface LS2

| LS2out.Luminosity | LS2out.SIGNAL_ASPECT |
| --- | --- |
| DAY | SIGNAL_ASPECT_1 |
| DAY | SIGNAL_ASPECT_2 |
| DAY | MOST_RESTRICTIVE_ASPECT |
| NIGHT | SIGNAL_ASPECT_1 |
| NIGHT | SIGNAL_ASPECT_2 |
| NIGHT | MOST_RESTRICTIVE_ASPECT |

Not permitted combinations on “Train driver” interface LS2

| LS2out.Luminosity | LS2out.SIGNAL_ASPECT |
| --- | --- |
| DARK | SIGNAL_ASPECT_1 |
| DARK | SIGNAL_ASPECT_2 |
| DARK | MOST_RESTRICTIVE_ASPECT |
| DAY | NO_SIGNAL_ASPECT |
| NIGHT | NO_SIGNAL_ASPECT |

Remark: in case of

- Lamp failure

- Luminosity failure

- Intentionally dark requested in PDI_telegram ILS->Efes LS

- Consequently result on “Train driver” interface LS2 shall be

- LS2out.SIGNAL_ASPECT = NO_SIGNAL ASPECT

- LS2out.LUMINOSITY= DARK

General change in all test cases: naming NO_ASPECT changed to NO_SIGNAL ASPECT!

General rule: when LS2out.SIGNAL_ASPECT = NO_SIGNAL ASPECT also LS2out.LUMINOSITY= DARK

## Changes on particular test cases

### TC_Eu.LS.5241.01

- Step 2:

  - LS2out.SIGNAL_ASPECT = SIGNAL_ASPECT_2 changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Step 4:

  - Func. Pkg.: Option changed to Option LS5

- Step 5:

  - Func. Pkg.: Option changed to Option LS4

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.5128.01

- Step 2:

  - Added condition LS2out.SIGNAL_ASPECT = NO_SIGNAL ASPECT

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.5097.01

- Step 2:

  - Added condition LS2out.LUMINOSITY=DAY

### TC_Eu.LS.5044.01

- Step 2:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - changed postcondition LS2out.LUMINOSITY from DAY to DARK

### TC_Eu.LS.5157.01

- Step 5:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Step 7:

  - Func. Pkg.: Option changed to Option LS5

- Step 8:

  - Func. Pkg.: Option changed to Option LS4

- Step 9:

  - PDI_TELEGRAM Efes LS->ILS, byte 51=0xFF (0xFF Intentionally dark not applicable)

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.5261.01

- Step 4:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Step 6:

  - Func. Pkg.: Option changed to Option LS5

- Step 7:

  - Func. Pkg.: Option changed to Option LS4

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - LS2out.LUMINOSITY=DAY changed to DARK

### TC_Eu.LS.5138.01

- Preconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

- Step 2:

  - LS2out.SIGNAL_ASPECT = SIGNAL_ASPECT_1 changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Step 4:

  - Func. Pkg.: Option changed to Option LS5

- Step 5:

  - Func. Pkg.: Option changed to Option LS4

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.5341.01

- Step 3:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Step 5:

  - Func. Pkg.: Option changed to Option LS5

- Step 6:

  - Func. Pkg.: Option changed to Option LS4

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.5322.01

- Step 2:

  - Changed internal error type from IE.02 to IE.02a (day fault because current luminosity=day)

- Step 3:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - Added condition LS2out.LUMINOSITY=DARK

- Step 4:

  - Func. Pkg.: Option changed to Option LS5

- Step 5:

  - Func. Pkg.: Option changed to Option LS4

- Postconditions:

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

  - LS2out.LUMINOSITY=DAY changed to DARK

### TC_Eu.LS.4975.01

- Preconditions:

  - LS2out.SIGNAL_ASPECT = SIGNAL_ASPECT_1 changed to MOST_RESTRICTIVE_ASPECT

- Configuration

  - Added config parameter LUMINOSITY_DEFAULT=DAY

- Postconditions:

  - Removed LS2out.LUMINOSITY

  - LS2out.SIGNAL_ASPECT = SIGNAL_ASPECT_1 changed to MOST_RESTRICTIVE_ASPECT

### TC_Eu.LS.5147.01

- Configuration

  - Added config parameter CON_DOWNGRADE_MOST_RESTRICT=FALSE

- Preconditions:

  - Precondition LS2out.SIGNAL_ASPECT = changed from SIGNAL_ASPECT_1 to MOST_RESTRICTIVE_ASPECT

- Step 1:

  - Receive command indicate SIGNAL_ASPECT_1 instead of SIGNAL_ASPECT_2

- Step 3:

  - Set to National Error Aspect SIGNAL_ASPECT_2 instead of MOST_RESTRICTIVE_ASPECT

- Step 7

  - Message Indicated Signal Aspect to ILS: SIGNAL_ASPECT_2 instead of MOST_RESTRICTIVE_ASPECT

- Postconditions:

  - Postcondition LS2out.SIGNAL_ASPECT changed from  MOST_RESTRICTIVE_ASPECT  to SIGNAL_ASPECT_2

### TC_Eu.LS.5207.01

- Step 2:

  - Added condition LS2out.LUMINOSITY=DAY

### TC_Eu.LS.5261.01

- Step 4:

  - Added condition LS2out.LUMINOSITY=DAY

- Postconditions:

  - LS2out.LUMINOSITY=DAY changed to DARK

### TC_Eu.LS.7953.01

- Preconditions:

  - LS2out.LUMINOSITY=DAY changed to DARK

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.5293.01

- Step 4:

  - Added condition LS2out.LUMINOSITY=DARK

  - LS2out.SIGNAL_ASPECT = NO_ASPECT changed to NO_SIGNAL ASPECT

### TC_Eu.LS.7777.01

- Step 2:

  - Changed type of IE.02 to IE.02b (luminosity night fault)

### Change Option to Option LS4 and Option LS5 - multiple test cases

Some steps in LS test cases are optional because LS4 and LS5 are optional interfaces on EfES. Due to this fact existing value Option in related test steps were renamed from Option to Option LS4 and Option LS5. Changes applied on following test cases:

- TC_Eu.LS.4996.01

- TC_Eu.LS.5016.01

- TC_Eu.LS.5024.01

- TC_Eu.LS.5060.01

- TC_Eu.LS.5079.01

- TC_Eu.LS.5097.01

- TC_Eu.LS.5138.01

- TC_Eu.LS.5147.01

- TC_Eu.LS.5157.01

- TC_Eu.LS.5207.01

- TC_Eu.LS.5241.01

- TC_Eu.LS.5251.01

- TC_Eu.LS.5261.01

- TC_Eu.LS.5272.01

- TC_Eu.LS.5293.01

- TC_Eu.LS.5322.01

- TC_Eu.LS.5341.01

- TC_Eu.LS.7951.01

- TC_Eu.LS.7952.01

- TC_Eu.LS.7953.01

# SubS Generic IO

## Changes of timing requirements - multiple test cases

In feedback to TC_Eu.IO.520  it was received that  condition "t1 + P_CON_T_ACTIVATION_DELAY_OFF < t; t2" is wrong, if the Generic IO switches the output in 5h, it is also fulfilled. The check shall be something like "t1 + P_CON_T_ACTIVATION_DELAY_OFF < t < t1 + P_CON_T_ACTIVATION_DELAY_OFF + 300 ms; t2", see also Eu.IO.1549 and other cases.

Based on feedback timing condition wwas improved to be like "t1 + P_CON_T_ACTIVATION_DELAY_OFF < t < t1 + P_CON_T_ACTIVATION_DELAY_OFF + 300 ms; t2". . Correction was applied to following test cases:

- TC_Eu.IO.1611.01

- TC_Eu.IO.1718.01

- TC_Eu.IO.1730.01

- TC_Eu.IO.1788.01

- TC_Eu.IO.497.01

- TC_Eu.IO.506.01

- TC_Eu.IO.510.01

- TC_Eu.IO.520.01

- TC_Eu.IO.522.01

## Activation delay - multiple test cases

In feedback to TC_Eu.IO.520  we received that P_CON_T_ACTIVATION_DELAY_OFF and P_CON_T_ACTIVATION_DELAY_ON are channel specific configuration values, on test cases value is defined once fore all channels. Also, both are configured to 1000s, it would be interesting to have different values.

Test case configuration was enhanced to be channel specific and different values. Following values are defined:

- P_CON_T_ACTIVATION_DELAY_OFF [1]: 1100 ms

- P_CON_T_ACTIVATION_DELAY_OFF [2]: 1300 ms

- P_CON_T_ACTIVATION_DELAY_OFF [3]: 1500 ms

- P_CON_T_ACTIVATION_DELAY_ON [1]: 100 ms

- P_CON_T_ACTIVATION_DELAY_ON [2]: 300 ms

- P_CON_T_ACTIVATION_DELAY_ON [3]: 500 ms

Changes were applied to:

- TC_Eu.IO.1611.01

- TC_Eu.IO.1718.01

- TC_Eu.IO.1730.01

- TC_Eu.IO.1788.01

- TC_Eu.IO.497.01

- TC_Eu.IO.506.01

- TC_Eu.IO.510.01

- TC_Eu.IO.520.01

- TC_Eu.IO.522.01

## Other changes on particular test cases

### TC_Eu.IO.495.01

- Preconditions

  - bug correction: IO3.CHANNEL_02_STATE value SWITCHED_OFF changed to SWITCHED_ON (was not aligned with RIC and VIC values)

  - IO2.CHANNEL_02_DISTURBANCE changed from PHYSICALLY_DISTURBED to NOT_PHYSICALLY_DISTURBED

  - bug correction: IO2.CHANNEL_02_STATE changed from SWITCHED_OFF to DISTURBED (antivalent)

- Step 7:

  - Change on byte 45 (OUT ch 2) from 0x02 (disturbed) to 0x01 (not disturbed)

- Postconditions

  - bug correction: IO3.CHANNEL_02_STATE value SWITCHED_OFF changed to SWITCHED_ON (was not aligned with RIC and VIC values)

  - bug correction: IO2.CHANNEL_02_STATE changed from SWITCHED_OFF to DISTURBED (antivalent)

### TC_Eu.IO.497.01

- Postconditions

  - States of output channels corrected to be in accordance with the “safe state”, which is “all physical channels off” (see Eu.SAS.2010).

### TC_Eu.IO.1718

- Step 3:

  - title of step 3 was changed, new text is: "State of output channel - no state change (channel 2 - OFF)"

### TC_Eu.IO.1730

- Step 2:

  - Bug: Value of byte 01..02 representing MessageType was changed from 0x0003 to 0x0001 (Set Output Channels)

- Step 3:

  - title of step 3 was changed, new text is: "State of output channel - no state change (channel 3 - ON)"

### TC_Eu.IO.7688, TC_Eu.IO.7689

- Test cases 7688 and 7689 redesigned to contains testing fixed 5 flashing cycles

- Steps 2, 3, 4

  - IO2.CHANNEL_03_TYPE value EQUIVALENT was corrected to SINGLE

# SubS Level Crossing

<No corrections applied>

# SubS TDS

## Changes on particular test cases

### TC_Eu.TDS.1954.01

- Step 1: changed values in PDI telegram

  - Byte 47: from 0x01 to 0xFF (POM status, 0xFF POM Status is not applicable)

  - Byte 48: from 0xFF to 0x01 (Disturbance status, 0x01 Disturbance is operational)

### TC_Eu.TDS.5979.01

- Step 2: changed values in PDI telegram

  - Byte 47: from 0x01 to 0xFF (POM status, 0xFF POM Status is not applicable)

### TC_Eu.TDS.5983.01

- Step 1: bug in step name

  - changed last part of step name from …(wheel_in) to …(wheel_out)

### TC_Eu.TDS.6001.01

- Step 2: bug in PDI telegram byte 49 (PDI_CHANGE_TRIGGER)

  - Byte 49 value changed from 0x01 to 0x02 (0x02 Command from EIL accepted)

- Step 3: step deleted duet o bug, with existing precondition last step - step3 cannot be executed

### TC_Eu.TDS.6003.01

- Postconditions

  - Bug correction: TVPS_OCCUPANCY_STATES changed to VACANT.

### TC_Eu.TDS.5987.01

- Step 8: bug in PDI telegram byte 49 (PDI_CHANGE_TRIGGER)

  - Byte 49 value changed from 0x01 (0x01 Passing detected) to 0x06 (0x06 Internal trigger)

### TC_Eu.TDS.6014.01

- Added new precondition parameter TVPS_FILLING_LEVEL with value 1

- Step 2: Disturbance status value corrected (byte 48) from 0x01 (Disturbance is operational)
to 0xFF (Disturbance status is not applicable).

### TC_Eu.TDS.7250.01

- Postconditions

  - Bug correction: TVPS_OCCUPANCY_STATES changed from OCCUPIED to DISTURBED_WITH_OPERATIONAL_REASON.

# SubS Point

## Changes on particular test cases

### TC_Eu.P.6761.01

- postconditions corrected to

  - P3in.ABILITY_TO_MOVE[1]=UNABLE

  - P3in.ABILITY_TO_MOVE[2]=UNABLE

### TC_Eu.P.3093.01

- TC was updated/enhanced that it start with "point is moving btw end positions" when situation defined by SD (NO_OPERATING_VOLTAGE) happens.

- P3in.ABILITY_TO_MOVE[1] was removed from pre and postconditions since it is not relevant for this TC.

### TC_Eu.P.1469.01  and TC_Eu.P.5371.01

- Comment added to test case description in order to clarify context of test case:

- comment: The intention of this particular test case is to test that there is no reaction from the SubS Point when it receive command from ILS. It should not send any telegram or start any movement as reaction to the received command. Postconditions are same as preconditions.

### TC_Eu.P.7098.01

- Step 7:

  - Change type/kind of test step from PDI_TELEGRAM  to LOGICAL_STATE. This step basicaly represent step 4 from sequence diagram